README

# Connection

| ESP32 | CONNECT TO |
| --- | --- |
| GND | GND |
| VCC | 3.3V |
| GPIO4 (RX1) | AVR TX |
| GPIO5 (TX1) | AVR RX |
| GPIO1(TX0) | Debug RX |
| GPIO3(RX0) | Debug TX |

UART 0: 9600b/s 8N1

UART 1: 9600b/s 8N1

# Communication ESP32  AVR

ESP32 expects data from AVR in from showed below:

Sensor_Name:Sensor_Value

Ex. Temperature:24

Data should be transmitted with appropriate delays:

Max. 0.5Hz (once per 2 seconds)

After receiving data ESP32 sends data to http server via GET query and sends back http code to AVR UART

Ex. 200 – OK, 404 – address not found, -1 connection timeout

Every negative value means connection error (possible errors available here)

# Communication ESP32  HTTP Server

ESP32 sends GET query with from showed below:

http://" +  serverIP + ":" + serverPORT + "/update-sensor?" + type + "=" + val

After sending GET query it’s waiting for server response and sends it to both UART0 and UART1.

POST query is not yet implemented.

# Program algorithm